Model: W728ADZ56FA

Dual light source 8MP@30fps 4K UHD Bullet POE IP camera

Features:

- 1/2.8" Sony CMOS sensor

- 4K resolution 3840 x 2160P

- Low illumination 0.01Lux

- Day/Night, AWB/MWB,BLC, HLC, 2D/3D-DNR,Shutter, IR-CUT

- D-WDR,Mirror, rotate, Defogging

- Multiply Style OSD Overlay,Motion Detection, Privacy Mask

- RTSP and Onvif 17.06 compatible with many third-party NVR

- H.264/H.265 dual-stream media server

- Easy-to-use P2P Cloud service

- Support user authentication, video data encryption

- Support Alarm Snapshot to Email and FTP

- Multiply WEB browser IE,Firefox 32bit ESR, PC Client, Mobile APP remote access

- RJ45 100mbps network port, IEEE802.3af PoE

- Lightning protection 4000V

- Strong water-proof housing, IP67

- 4PCS IR& Warm dual Light LEDs,Light Distance:30-40M

- 2.8mm F1.6 fixed lens

- Built in microphone optional

Technical Specifications:

| Model | W728ADZ56FA |
| --- | --- |
| Camera |  |
| Image Sensor | 1/2.8" Sony CMOS |
| Effective Pixels | 8 megapixels |
| Electronic Shutter | AUTO, 1/25s ~ 1/100000s |
| Min. Illumination | 0.01Lux@F1.2(AGC ON), 0Lux IR on |
| Day/Night | Auto/Color/(B/W)/Timing |
| WDR | Digital WDR |
| White Balance | Auto/Manual |
| BLC /HLC | Support |
| DNR | 2D/3D DNR |
| License Plate Mode | Yes |
| Other | Human body and Vehicle detection,AGC,Motion Detection,Privacy Mask,Flip,Rotate,,OSD overlay |
| Encode |  |
| Video Standard | H.264/H.265 |
| Video Resolution | MainStream:30fps@8MP(3840x2160)/6MP(3072x2048)/5MP(2592x1944)/4MP/3MP/1080P/720P,etc. Sub stream: 720x480/VGA(640x480)/360P/QVGA@30fps,etc. Third stream:1080P/720P/CIF@10fps |
| Video Bitrates | 512Kbps - 12Mbps, VBR/CBR |
| Audio Standard | G.711-u/G.711-a/AAC |
| Built-in Mic | Optional |
| Built-in Speaker | N/A |
| SD card slot | Optional |
| OSD Overlay | Title and Time overlay ,custom color adjustable |
| LED |  |
| Led | 4PCS IR& Warm dual Light LEDs |
| Distance | 30-40 M |
| Lens |  |
| Focal length | 2.8mm F1.6 fixed lens |
| Language |  |
| English, simplified Chinese, Chinese Traditional, Deutsch, Italian, Turkish, Polish, Korean, Japanese, Russian, Ukrainian, Persian. |  |
| Network Services |  |
| Protocol | HTTP/RTSP/FTP/SMTP/DHCP/NTP/NFS, etc. |
| P2P | Yes |
| Web | IE , Firefox,Edge, Google Chrome etc. |
| Media | CMS, Android, IOS |
| ONVIF | 17.06 compatible |
| General |  |
| Network Port | 1-RJ45, 100Mbps, POE |
| Power Supply | 12 VDC ± 10%,POE |
| Power Consumption | < 8 W |
| Operating Temp. | -30℃-(+60)℃, 10%-90%RH |